# INSTALLATION & DEPLOYMENT GUIDE

Cyber-Physical Smart Building Access Control System

# 1. Components & Bring-Up Order

The system has five independently deployable pieces. Bring them up in this order — each one depends on the previous being reachable, and starting out of order just produces confusing "not connected yet" log lines rather than an actual failure:

| # | Component | Depends on |
| --- | --- | --- |
| 1 | MQTT broker (Mosquitto, HiveMQ, or similar) | Nothing — this is the hub everything else connects to |
| 2 | Backend (FastAPI + database) | The broker (to receive live status/events); the database can be SQLite with zero setup |
| 3 | Building Gateway (only if any doors use the RS-485 backbone) | The broker + the RS-485 bus being wired |
| 4 | Door node firmware (on each ESP32) | Wi-Fi + broker reachable directly, OR the Gateway if using RS-485 |
| 5 | Dashboard (React SPA) | The backend's REST API |

# 2. MQTT Broker

Any MQTT 3.1.1 broker works — Mosquitto is the common choice for a real deployment. For development/testing without installing anything system-wide, a pure-Python broker works too:

pip install amqtt
amqtt   # binds 0.0.0.0:1883, anonymous auth allowed by default

Production note: enable authentication (username/password or client certificates) and TLS on the broker before deploying — the backend, gateway, and firmware all support MQTT_USE_TLS / credentials, but the broker has to actually require them.

# 3. Backend

cd backend
python3 -m venv venv && source venv/bin/activate
pip install -r requirements.txt
cp .env.example .env          # edit DATABASE_URL, JWT_SECRET, MQTT_* as needed
python -m scripts.seed_db      # creates tables + demo users/doors/credentials
uvicorn app.main:app --host 0.0.0.0 --port 8000

Required .env values for a real deployment (the shipped defaults are dev-only placeholders):

| Variable | Notes |
| --- | --- |
| DATABASE_URL | SQLite by default (zero setup); switch to PostgreSQL for anything beyond a pilot — see Section 7 |
| JWT_SECRET | Long random string; rotating it invalidates all existing sessions |
| CREDENTIAL_ENCRYPTION_KEY / CREDENTIAL_INDEX_KEY | Fernet key + HMAC key for credential encryption at rest (Phase 5). Must be set to real random values and persisted — losing them makes stored card UIDs permanently unreadable |
| MQTT_BROKER_HOST / PORT / USE_TLS / USERNAME / PASSWORD | Point at the broker from Section 2 |
| DOOR_STALE_AFTER_SECONDS / DOOR_STALENESS_CHECK_INTERVAL_SECONDS | Phase 7 staleness watchdog tuning — defaults (30s / 10s) are reasonable for a single building |

Visit http://<host>:8000/docs to confirm the API is up and browse every endpoint interactively (see API_Reference.docx for the full written reference).

# 4. Building Gateway (only if using the RS-485 backbone)

Skip this section entirely if every door node connects directly over Wi-Fi. Deploy this if doors share an RS-485 multi-drop bus (see Phase6_Multi_Node_Deployment_Guide.docx for the wiring/addressing details).

cd gateway
pip install -r requirements.txt
cp gateway_config.example.yaml gateway_config.yaml
# edit: serial.port (the USB-RS485 adapter's device path),
#       nodes: (addr -> door code mapping, must match each node's config.h)
python rs485_gateway.py --config gateway_config.yaml

The gateway needs no changes to the backend or database — a gateway-relayed door looks identical to a directly-connected one on the MQTT topics both use.

# 5. Door Node Firmware (per ESP32)

cd door_node_firmware
cp include/secrets_example.h include/secrets.h   # fill in Wi-Fi + broker credentials
# edit include/config.h for THIS specific node:
#   DOOR_ID, DOOR_REQUIRES_BIO, DOOR_FAIL_MODE_SAFE
#   RS485_NODE_ADDR (only relevant if this node is on the RS-485 backbone)
#   DESFIRE_AES_KEY (replace the placeholder before provisioning real cards)
pio run --target upload
pio device monitor

Wire per Phase2_Wiring_and_Bench_Test_Guide.docx (single node) and, if applicable, Phase6_Multi_Node_Deployment_Guide.docx (RS-485 bus topology/addressing for multiple nodes). Run the Phase 2 bench test checklist on every node before it goes into a real door.

Important: this firmware has been reviewed line-by-line but never compiled/run on real ESP32 hardware in this project's development environment (no hardware, no working ESP32 toolchain available there). Treat pio run as your first real verification step, not this document.

# 6. Dashboard

cd dashboard
npm install
echo "VITE_API_BASE_URL=http://<backend-host>:8000" > .env
npm run build      # production build -> dist/
# serve dist/ with any static file server (nginx, serve, etc.)
# OR for local development only:
npm run dev

The dashboard is a static SPA — it can be hosted anywhere that can serve static files and reach the backend's REST API. Set VITE_API_BASE_URL to wherever the backend actually runs; it defaults to http://localhost:8000 for local development.

# 7. Database: SQLite vs. PostgreSQL

SQLite is the zero-setup default and was load-tested at pilot scale (16 concurrent simulated nodes, 0 dropped events — see the Phase 6 deployment guide) without issue. For anything beyond a pilot, switch DATABASE_URL to PostgreSQL:

DATABASE_URL=postgresql+psycopg2://user:password@host:5432/access_control

No code changes are needed — the backend uses SQLAlchemy throughout. Install psycopg2-binary (not in requirements.txt by default, to keep the zero-setup SQLite path dependency-free) and re-run python -m scripts.seed_db against the new database.

# 8. Post-Install Checklist

- GET /health on the backend returns { "status": "ok" }.

- Log into the dashboard with the seeded admin account (or a real one you've created via POST /api/users) and confirm the door list loads.

- Power on one door node (or the gateway + its nodes) and confirm its status flips to online in the dashboard within a few seconds.

- Present a provisioned card at that door and confirm an access event appears in that door's log.

- Trigger the tamper switch and confirm an alert appears and the door force-relocks.

- Change the default admin/instructor passwords and rotate JWT_SECRET / CREDENTIAL_ENCRYPTION_KEY / CREDENTIAL_INDEX_KEY away from the repository's example values before going anywhere near production data.