# PHASE 5 — SECURITY REVIEW

Cyber-Physical Smart Building Access Control System

# 1. Scope and Method

This is an internal design and code review (P5.5), not a live penetration test — there is no deployed hardware or network to attack in this environment. The review covers the firmware (door_node_firmware/), backend (backend/), and dashboard (dashboard/) as delivered through Phase 5, checked against the threats identified in the original proposal review and the System Design Document's security section.

Where a mitigation has been implemented, this document says so and points at the file/test that proves it. Where it hasn't, that is stated plainly rather than implied away — the goal is an accurate picture for the committee and for whoever picks up Phase 6, not a clean-looking checklist.

# 2. Threat Model Summary

| Asset | Threat actor / scenario | Primary risk |
| --- | --- | --- |
| Physical door lock | Someone presents a cloned/emulated card UID | Unauthorized entry |
| Door node hardware | Someone opens the enclosure to tamper with wiring | Bypass of lock control, sensor spoofing |
| Credential database | Attacker gains DB read access (backup leak, SQL injection, etc.) | Mass credential/UID exposure |
| MQTT broker traffic | Network eavesdropper on the door-to-broker link | Credential/event interception |
| Admin dashboard/API | Credential stuffing or brute force against /api/auth/login | Account takeover |
| Door node firmware | Physical access to a node, dumping flash | Extraction of the shared AES key, Wi-Fi credentials |

# 3. Before / After — Gaps Flagged Since the Original Proposal Review

| Gap flagged | Status | What changed |
| --- | --- | --- |
| Card cloning — UID-only read, no cryptographic proof of card identity | Mitigated | desfire_auth.cpp implements DESFire EV1 AES-128 mutual authentication; CARD_SCAN now requires it to pass, not just a UID match |
| Credential data stored in plaintext at rest | Mitigated | app/crypto.py — Fernet-encrypted card_uid + keyed HMAC blind index for lookup (backend/tests/test_crypto.py) |
| No login rate limiting / brute-force protection | Mitigated | app/rate_limit.py — per-account lockout after repeated failures (backend/tests/test_auth.py) |
| Tamper detection only alerted, took no protective action | Mitigated | Tamper interrupt now forces immediate relock and blocks further card entry until admin-cleared via MQTT |
| No local resilience if MQTT/network drops mid-operation | Mitigated | Offline event ring buffer (queueOfflineEvent/flushOfflineEvents) — events survive a broker outage, RAM-only |
| Fire/life-safety egress override on fail-secure doors | Mitigated (Phase 1) | REX button + mechanical key override on every door, independent of firmware state; confirmed still functional during tamper lockout |
| Per-card key diversification | Not mitigated | All cards at a site share one AES key (DESFIRE_AES_KEY) — see Section 4 |
| MQTT topic/broker access control per device | Partial | TLS + username/password supported in both firmware and backend config; per-device certificate-based auth not implemented |
| JWT revocation / session management | Not mitigated | Tokens are valid until expiry (60 min default) with no revocation list — logging out client-side doesn't invalidate a leaked token early |
| Firmware secure boot / code signing | Not mitigated | Out of scope for this project's timeline — flagged as a real gap for a production deployment, not something addressed here |

# 4. Residual Risks and Recommendations

## 4.1 Shared AES key across all cards

Every door node currently authenticates cards against the same DESFIRE_AES_KEY. If that key is ever extracted from one node's flash (physical access + flash dump), every card at the site is compromised, not just that node's. The standard mitigation is per-card key diversification — deriving each card's key from a master key and the card's UID via AES-CMAC (documented in NXP's application notes) — which was not implemented here due to time/complexity, and is the single highest-value next security investment if this project continues past Phase 5.

## 4.2 DESFire handshake is unverified against real hardware

The AES mutual-authentication implementation was written against the publicly documented DESFire EV1 protocol and reviewed line-by-line, but the ESP32 toolchain could not be compiled in this environment (no hardware, limited sandboxed network access to the Espressif platform download), so it has never actually run against a real card. Byte-order or framing mistakes in a hand-implemented crypto handshake are exactly the kind of bug that looks correct on paper and fails — or worse, silently accepts something it shouldn't — on real hardware. This must be bench-tested against real DESFire cards before any reliance on it.

## 4.3 Encryption key management

CREDENTIAL_ENCRYPTION_KEY and CREDENTIAL_INDEX_KEY fall back to a deterministic dev-only key derived from a fixed string if unset, so the project runs out of the box without extra setup. This is safe for development but must never happen in a real deployment — both keys must be set to real random values and kept outside version control, exactly like JWT_SECRET already is. Losing or rotating CREDENTIAL_ENCRYPTION_KEY without a migration plan makes existing encrypted card UIDs permanently unreadable, which is worth a deliberate key-rotation runbook before going further.

## 4.4 Rate limiting is per-process, in-memory

app/rate_limit.py tracks failed logins per process. A single backend instance (this project's target) is fine; a multi-instance deployment behind a load balancer would need a shared store (Redis, or a DB-backed table) so attempts are tracked consistently across instances.

## 4.5 Offline event buffer is RAM-only

Buffered access events are lost if a door node loses power while offline (not just network-down) — they exist only in RAM, not flash. Persisting the queue to LittleFS would close this gap if audit completeness during extended outages becomes a requirement.

# 5. Design/Code Review Checklist

| Check | Status | Notes |
| --- | --- | --- |
| Passwords hashed, never stored/logged in plaintext | Mitigated | bcrypt via app/security.py, verified in backend tests |
| Credential secrets (card UIDs) encrypted at rest | Mitigated | app/crypto.py, Fernet + blind index |
| Transport encryption for MQTT | Mitigated (config-level) | TLS supported in firmware (WiFiClientSecure) and backend (MQTT_USE_TLS); depends on being enabled against a real broker with a real cert, not self-signed/skipped |
| Role-based access control enforced server-side, not just hidden in UI | Mitigated | require_admin dependency in FastAPI routes; dashboard's UI restrictions are cosmetic on top of this, not the actual boundary |
| Brute-force protection on authentication | Mitigated | app/rate_limit.py |
| Tamper response beyond alerting | Mitigated | Forced relock + lockout, Phase 5 firmware |
| Life-safety egress not blocked by security logic | Mitigated | REX button bypasses tamper lockout and credential checks entirely |
| Cryptographic (not just UID) card authentication | Mitigated, unverified on hardware | See Section 4.2 |
| Per-credential key diversification | Not mitigated | See Section 4.1 |
| Audit log resilience to network/power loss | Partial | Network loss covered (offline buffer); power loss to a node is not (RAM-only) |
| Session/token revocation | Not mitigated | See Section 3 |
| Firmware code signing / secure boot | Not mitigated | Out of scope for this project |

# 6. Conclusion

Phase 5 closes the specific gaps flagged earliest and most repeatedly in this project's review history: cloneable card credentials, plaintext data at rest, no brute-force protection, and tamper detection with no teeth. What remains open — key diversification, hardware-verified crypto, session revocation, firmware code signing — are reasonable boundaries for a graduation project's scope, but each is named explicitly here rather than left implicit, so a reader (or a future phase) knows exactly what "security" does and doesn't mean for this system as it stands.